Name : Benjamin Danish D

Reg No : 192412544

Course name : Advanced Wireless Communication System

Course code : ECA5610

MATLAB Analysis of Spectrum Bands, Propagation Loss and Cellular Interference

Aim:

To compare a simplified free-space path-loss estimate across representative sub-1 GHz, mid-band and mmWave frequencies, and examine how distance and frequency reuse affect received signal and a simple interference measure.

Objectives:

- Calculate or simulate path loss at representative sub-1 GHz, mid-band and mmWave frequencies.

- Compare estimated received signal strength as frequency and distance change.

- Examine how cell separation or frequency reuse changes a simplified interference ratio.

- Relate the results to coverage and spectrum-use considerations across mobile generations, while stating model limitations.

Requirements:

- Computer with MATLAB R2019b or later.

- No RF hardware is required.

Program — Path Loss, Received Power and Reuse Interference

clear;

clc;

close all;

% Representative frequency bands

freqGHz = [0.7 3.5 28];

freqMHz = freqGHz * 1000;

bandName = ["Sub-1 GHz"; "Mid-band"; "mmWave"];

% Link budget parameters

txPowerdBm = 30;

gtDbi = 0;

grDbi = 0;

distanceKm = logspace(log10(0.1), log10(2), 200);

distanceAtPointKm = 0.5;

% Free-space path loss function

fspl = @(fMHz,dKm) 32.44 + 20*log10(fMHz) ...

+ 20*log10(dKm);

% Calculate path loss and received power at 0.5 km

lossAtPoint = fspl(freqMHz, distanceAtPointKm);

rxAtPoint = txPowerdBm + gtDbi + grDbi - lossAtPoint;

linkTable = table(bandName, freqGHz(:), ...

lossAtPoint(:), rxAtPoint(:), ...

'VariableNames', {'Band','Frequency_GHz', ...

'FSPL_dB','EstimatedRx_dBm'});

disp('Path Loss and Received Power at 0.5 km:');

disp(linkTable);

% Plot path loss and received power

figure;

tiledlayout(1,2);

% Free-space path loss

nexttile;

hold on;

grid on;

for k = 1:numel(freqMHz)

plot(distanceKm, fspl(freqMHz(k), distanceKm), ...

'LineWidth', 1.5);

end

set(gca, 'XScale', 'log');

xlabel('Distance (km)');

ylabel('Path Loss (dB)');

legend(bandName, 'Location', 'northwest');

title('Free-Space Path Loss');

hold off;

% Received power

nexttile;

hold on;

grid on;

for k = 1:numel(freqMHz)

rx = txPowerdBm + gtDbi + grDbi ...

- fspl(freqMHz(k), distanceKm);

plot(distanceKm, rx, 'LineWidth', 1.5);

end

set(gca, 'XScale', 'log');

xlabel('Distance (km)');

ylabel('Estimated Received Power (dBm)');

legend(bandName, 'Location', 'southwest');

title('Illustrative Received Power');

hold off;

% Frequency reuse and co-channel interference

cellRadiusKm = 0.5;

reuseN = [1 3 7 12];

pathLossExponent = 3.5;

reuseDistance = sqrt(3*reuseN) * cellRadiusKm;

sirLinear = (reuseDistance/cellRadiusKm) ...

.^pathLossExponent / 6;

sirDb = 10*log10(sirLinear);

% Display reuse results

reuseTable = table(reuseN(:), reuseDistance(:), sirDb(:), ...

'VariableNames', {'ReuseClusterN', ...

'ApproxInterfererDistance_km','ApproxSIR_dB'});

disp('Frequency Reuse and SIR Results:');

disp(reuseTable);

% Plot SIR versus reuse cluster size

figure;

plot(reuseN, sirDb, 'o-', 'LineWidth', 1.5);

grid on;

xlabel('Reuse Cluster Size N');

ylabel('Approximate SIR (dB)');

title('Co-Channel SIR versus Reuse Cluster Size');

% Display received power summary

fprintf('Estimated received powers at %.2f km:\n', ...

distanceAtPointKm);

disp(table(bandName, rxAtPoint(:), ...

'VariableNames', {'Band','Rx_dBm'}));

Simulation Outputs (Free-Space Path Loss and Received Power plots):

Simulation Outputs (Co-Channel SIR versus Reuse Cluster Size):

Results to Record:

| Band | Frequency (GHz) | FSPL at 0.5 km (dB) | Estimated Rx (dBm) |
| --- | --- | --- | --- |
| Sub-1 GHz | 0.7 | 83.321 | -53.321 |
| Mid-band | 3.5 | 97.301 | -67.301 |
| mmWave | 28 | 115.36 | -85.363 |

| Reuse N | Interferer distance (km) | Approx. SIR (dB) |
| --- | --- | --- |
| 1 | 0.86603 | 0.56811 |
| 3 | 1.5 | 8.9177 |
| 7 | 2.2913 | 15.357 |
| 12 | 3 | 19.454 |

Analysis Questions:

- At a fixed distance, which representative band has the highest free-space loss? Explain the frequency term in the equation.

- How does doubling distance affect FSPL in this model?

- What trend does the reuse-SIR estimate show as N increases, and which geometric assumption drives that trend?

- Why should the calculated received power not be treated as a reliable coverage radius for an actual cellular system?

- Relate the bands qualitatively to coverage and capacity trade-offs without treating this simple model as a comparison of complete mobile generations.

Conclusion:

Free-space path loss increases with both distance and frequency: at 0.5 km the mmWave band shows the highest loss and the sub-1 GHz band the lowest, and doubling distance or frequency adds about 6 dB. The illustrative SIR rises as the reuse cluster size N increases because the co-channel interferers are placed farther away. The free-space assumption, fixed transmit power and idealized six-interferer geometry are limitations; practical coverage depends on antenna characteristics, environment, bandwidth, receiver sensitivity, interference, regulatory constraints and the chosen propagation model.